PRACTICAL 10 — AAA AUTHENTICATION (UPDATED FOR YOUR REWIRED TOPOLOGY)

Based on your handwritten topology + the Practical 10 journal

I have changed the router LAN interface references to match your drawing: R1 G0/0 → S2 → PC-A, R2 G0/0 → S1 → PC-B + TACACS+ Server, and R3 G0/0 → S3 → PC-C + RADIUS Server. The serial links remain R1↔R2 and R2↔R3.

# 1. Final Topology / Address Plan

| Device / Interface | IP Address | Connection / Role |
| --- | --- | --- |
| R1 G0/0 | 192.168.1.1 /24 | S2 → PC-A LAN |
| R1 S0/0/0 | 10.1.1.2 /30 | Serial link to R2 S0/0/0 |
| R2 G0/0 | 192.168.2.1 /24 | S1 → PC-B + TACACS+ Server |
| R2 S0/0/0 | 10.1.1.1 /30 | Serial link to R1 |
| R2 S0/0/1 | 10.2.2.1 /30 | Serial link to R3 S0/0/1 |
| R3 G0/0 | 192.168.3.1 /24 | S3 → PC-C + RADIUS Server |
| R3 S0/0/1 | 10.2.2.2 /30 | Serial link to R2 |
| PC-A | 192.168.1.3 /24 | Gateway 192.168.1.1 |
| PC-B | 192.168.2.3 /24 | Gateway 192.168.2.1 |
| TACACS+ Server | 192.168.2.2 /24 | Gateway 192.168.2.1 |
| PC-C | 192.168.3.3 /24 | Gateway 192.168.3.1 |
| RADIUS Server | 192.168.3.2 /24 | Gateway 192.168.3.1 |

# 2. Packet Tracer PC / Server IP Configuration

Enter these values under Desktop → IP Configuration.

## PC-A

IP Address: 192.168.1.3

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

## PC-B

IP Address: 192.168.2.3

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

## TACACS+ Server

IP Address: 192.168.2.2

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.2.1

## PC-C

IP Address: 192.168.3.3

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.3.1

## RADIUS Server

IP Address: 192.168.3.2

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.3.1

# 3. Router 1 Configuration

Router>en

Router#conf t

Router(config)#host r1

r1(config)#int g0/0

r1(config-if)#ip address 192.168.1.1 255.255.255.0

r1(config-if)#no shut

r1(config-if)#int s0/0/0

r1(config-if)#ip address 10.1.1.2 255.255.255.252

r1(config-if)#no shut

# 4. Router 2 Configuration

Router>en

Router#conf t

Router(config)#host r2

r2(config)#int g0/0

r2(config-if)#ip address 192.168.2.1 255.255.255.0

r2(config-if)#no shut

r2(config-if)#int s0/0/0

r2(config-if)#ip address 10.1.1.1 255.255.255.252

r2(config-if)#no shut

r2(config-if)#int s0/0/1

r2(config-if)#ip address 10.2.2.1 255.255.255.252

r2(config-if)#no shut

# 5. Router 3 Configuration

Router>en

Router#conf t

Router(config)#host r3

r3(config)#int g0/0

r3(config-if)#ip address 192.168.3.1 255.255.255.0

r3(config-if)#no shut

r3(config-if)#int s0/0/1

r3(config-if)#ip address 10.2.2.2 255.255.255.252

r3(config-if)#no shut

# 6. Configure RIP

## Router 1

r1(config)#router rip

r1(config-router)#network 192.168.1.0

r1(config-router)#network 10.1.1.0

r1(config-router)#exit

r1#show ip route

## Router 2

r2(config)#router rip

r2(config-router)#network 10.1.1.0

r2(config-router)#network 192.168.2.0

r2(config-router)#network 10.2.2.0

r2(config-router)#exit

r2#show ip route

## Router 3

r3(config)#router rip

r3(config-router)#network 192.168.3.0

r3(config-router)#network 10.2.2.0

r3(config-router)#exit

r3#show ip route

# 7. Local AAA Authentication — Router 1

r1>en

r1#conf t

r1(config)#username kaanchi secret ty22

r1(config)#aaa new-model

r1(config)#aaa authentication login default local

r1(config)#line console 0

r1(config-line)#login authentication default

Login credentials: Username = kaanchi, Password = ty22.

# 8. Local AAA Authentication for VTY / SSH — Router 1

r1>en

r1#conf t

r1(config)#ip domain-name security.com

r1(config)#crypto key generate rsa

How many bits in the modulus? 1024

r1(config)#aaa authentication login SSH-LOGIN local

r1(config)#line vty 0 4

r1(config-line)#login authentication SSH-LOGIN

r1(config-line)#transport input ssh

r1(config-line)#exit

From PC-A Command Prompt:

ssh -l Kaanchi 192.168.1.1

Password: ty22

# 9. TACACS+ AAA — Router 2

TACACS+ Server: Services → AAA → turn service ON → set client IP/password and create the username/password.

r2>en

r2#conf t

r2(config)#username admin secret admin123

r2(config)#tacacs-server host 192.168.2.2

r2(config)#tacacs-server key pwdr2

r2(config)#aaa new-model

r2(config)#aaa authentication login default group tacacs+ local

r2(config)#line console 0

r2(config-line)#login authentication default

TACACS+ credentials: Username = admin, Password = admin123.

# 10. RADIUS AAA — Router 3

RADIUS Server: Services → AAA → turn service ON → set client IP/password and create the username/password.

r3>en

r3#conf t

r3(config)#username admin1 secret admin321

r3(config)#radius-server host 192.168.3.2

r3(config)#radius-server key pwdr3

r3(config)#aaa new-model

r3(config)#aaa authentication login default group radius local

r3(config)#line console 0

r3(config-line)#login authentication default

RADIUS credentials: Username = admin1, Password = admin321.

# 11. Quick Verification

PC-A> ping 192.168.2.3

PC-A> ping 192.168.3.3

PC-A> ssh -l Kaanchi 192.168.1.1

PC-C> ping 192.168.2.2

If a ping fails, first check interface status with `show ip interface brief`, then check RIP with `show ip route`.

# Important Interface Change From Your Original Journal

Original Practical 10 journal used R1 G0/1 and R3 G0/1 for their LANs. Your handwritten topology shows the LAN connections on G0/0, so this updated version uses R1 G0/0 and R3 G0/0. R2 already uses G0/0. The IP subnets are retained because they still fit the three LANs shown in your drawing.